SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

QUESTIONS: 5								Total Marks:50

Annexure A

Scenario-Based

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid host IP address range for each subnet.

- Calculate the number of usable host addresses available in each subnet.

- Determine the remaining unused IP address range within the given network block.

ANSWER :

Given

- Network: 192.168.100.0/24

- Computing: /25

- Electronics: /26

- Mechanical Engineering: /27

## Formulae Used

### 1. Number of host bits : host bits = 32 – prefix length

### 2. Total number of addresses : total address = 2host bits  - 2

### 3. Usable host addresses : 2host bits – 2

### 4. Usable hosts = 256 – last octet of subnet mask

School of Computing — /25

Subnet mask : /25 = 255.255.255.128

Step 1: Calculate host bits

Host bits = 32 – 25 = 7 bits

Step 2: Calculate total IP addresses

Total IP addresses = 2^7 = 128 addresses

Step 3: Calculate usable host addresses

Usable hosts = 128 – 2 = 126 hosts

Step 4: Calculate block size

Block size = 256 – 128 = 128

Therefore, the first subnet ranges from : 192.168.100.0 to 192.168.100.127

Therefore:

Network Address = 192.168.100.0

Valid Host Range = 192.168.100.1 to 192.168.100.126

Broadcast Address = 192.168.100.127

Usable Host Addresses = 126

School of Electronics – /26

The previous subnet ends at 192.168.100.127.

Therefore, the next available address is 192.168.100.128.

Subnet mask for /26 = 255.255.255.192

Host bits = 32 – 26 = 6 bits

Total IP addresses = 2^6 = 64 addresses

Usable hosts = 64 – 2 = 62 hosts

Block size = 256 – 192 = 64

Starting from 192.168.100.128, the subnet ranges from : 192.168.100.128 to 192.168.100.191

Therefore:

Network Address = 192.168.100.128

Valid Host Range = 192.168.100.129 to 192.168.100.190

Broadcast Address = 192.168.100.191

Usable Host Addresses = 62

School of Mechanical Engineering – /27

The previous subnet ends at 192.168.100.191.

Therefore, the next available address is 192.168.100.192.

Subnet mask for /27 = 255.255.255.224

Host bits = 32 – 27 = 5 bits

Total IP addresses = 2^5 = 32 addresses

Usable hosts = 32 – 2 = 30 hosts

Block size = 256 – 224 = 32

Starting from 192.168.100.192, the subnet ranges from : 192.168.100.192 to 192.168.100.223

Therefore:

Network Address = 192.168.100.192

Valid Host Range = 192.168.100.193 to 192.168.100.222

Broadcast Address = 192.168.100.223

Usable Host Addresses = 30

Remaining Unused IP Address Range

The original /24 network ends at : 192.168.100.255

The Mechanical Engineering subnet ends at : 192.168.100.223

Therefore, the next available address is : 192.168.100.224

Hence, the remaining unused IP range is : 192.168.100.224 to 192.168.100.255

FINAL ANSWER :

| Department | Subnet | Network Address | Valid Host IP Range | Broadcast Address | Usable Host Addresses |
| --- | --- | --- | --- | --- | --- |
| School of Computing | /25 | 192.168.100.0 | 192.168.100.1 – 192.168.100.126 | 192.168.100.127 | 126 |
| School of Electronics | /26 | 192.168.100.128 | 192.168.100.129 – 192.168.100.190 | 192.168.100.191 | 62 |
| School of Mechanical Engineering | /27 | 192.168.100.192 | 192.168.100.193 – 192.168.100.222 | 192.168.100.223 | 30 |

2. A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid range of host IP addresses.

- Calculate the number of usable hosts in each subnet.

- Identify the unused IP address range remaining within the allocated IP block.

ANSWERS :

Given

- Software Development → /24

- Quality Assurance → /25

- Human Resources → /26

- Executive Management → /27

Formulae Used

Host bits = 32 − Prefix length

Total addresses = 2^(Host bits)

Usable hosts = Total addresses − 2

Block size = 256 − Last octet of subnet mask

Software Development – /24

Subnet mask = 255.255.255.0

Host bits = 32 − 24 = 8

Total addresses = 2⁸ = 256

Usable hosts = 256 − 2 = 254

Block size = 256 − 0 = 256

Therefore:

- Network Address = 10.10.0.0

- Valid Host Range = 10.10.0.1 – 10.10.0.254

- Broadcast Address = 10.10.0.255

- Usable Hosts = 254

Quality Assurance – /25

The previous subnet ends at 10.10.0.255, so the next available address is 10.10.1.0.

Subnet mask = 255.255.255.128

Host bits = 32 − 25 = 7

Total addresses = 2⁷ = 128

Usable hosts = 128 − 2 = 126

Block size = 256 − 128 = 128

Therefore:

- Network Address = 10.10.1.0

- Valid Host Range = 10.10.1.1 – 10.10.1.126

- Broadcast Address = 10.10.1.127

- Usable Hosts = 126

Human Resources – /26

The previous subnet ends at 10.10.1.127, so the next available address is 10.10.1.128.

Subnet mask = 255.255.255.192

Host bits = 32 − 26 = 6

Total addresses = 2⁶ = 64

Usable hosts = 64 − 2 = 62

Block size = 256 − 192 = 64

Therefore:

- Network Address = 10.10.1.128

- Valid Host Range = 10.10.1.129 – 10.10.1.190

- Broadcast Address = 10.10.1.191

- Usable Hosts = 62

Executive Management – /27

The previous subnet ends at 10.10.1.191, so the next available address is 10.10.1.192.

Subnet mask = 255.255.255.224

Host bits = 32 − 27 = 5

Total addresses = 2⁵ = 32

Usable hosts = 32 − 2 = 30

Block size = 256 − 224 = 32

Therefore:

- Network Address = 10.10.1.192

- Valid Host Range = 10.10.1.193 – 10.10.1.222

- Broadcast Address = 10.10.1.223

- Usable Hosts = 30

FINAL ANSWER :

| Department | Subnet | Network Address | Valid Host IP Range | Broadcast Address | Usable Host Addresses |
| --- | --- | --- | --- | --- | --- |
| Software Development | /24 | 10.10.0.0 | 10.10.0.1 – 10.10.0.254 | 10.10.0.255 | 254 |
| Quality Assurance | /25 | 10.10.1.0 | 10.10.1.1 – 10.10.1.126 | 10.10.1.127 | 126 |
| Human Resources | /26 | 10.10.1.128 | 10.10.1.129 – 10.10.1.190 | 10.10.1.191 | 62 |
| Executive Management | /27 | 10.10.1.192 | 10.10.1.193 – 10.10.1.222 | 10.10.1.223 | 30 |

| Original IP Block | Allocated Up To | Remaining Unused IP Range |
| --- | --- | --- |
| 10.10.0.0/16 | 10.10.1.223 | 10.10.1.224 – 10.10.255.255 |

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

- Convert the given IPv6 address into its compressed notation.

- Expand the compressed address back into its full hexadecimal representation.

- Identify the network prefix using the /64 prefix length.

- Calculate the total number of host addresses supported within the subnet.

- the total number of possible host addresses available within this network.

ANSWER :

Given IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

Compressed notation

Remove leading zeros in each group and replace the consecutive groups of all zeros with ::.

Answer: 2001:db8:abcd::/64

Expanded notation

The compressed address : 2001:db8:abcd::/64

contains three written groups followed by ::.

:: represents five groups of 0000 here.

Therefore : 2001:0db8:abcd:0000:0000:0000:0000:0000/64

Network Prefix

A /64 means the first 64 bits are the network portion.

The first four groups contain 64 bits : 2001:0db8:abcd:0000

Therefore: Network Prefix = 2001:0db8:abcd:0000/64

Host portion = remaining 64 bits

Total Host Addresses

IPv6 has 128 bits.

Network bits = 64

Host bits: 128 − 64 = 64 bits

Formula: Total host addresses = 2^(host bits)

= 2^64 = 18,446,744,073,709,551,616

Total Possible Host Addresses

Since the question asks for the total possible host addresses within this /64 network, it is:

2^64 = 18,446,744,073,709,551,616 addresses

Unlike IPv4, we do not subtract 2 for network and broadcast addresses in the usual IPv6 host-count calculation.

| Question | Answer |
| --- | --- |
| 1. Compressed notation | 2001:db8:abcd::/64 |
| 2. Expanded notation | 2001:0db8:abcd:0000:0000:0000:0000:0000/64 |
| 3. Network prefix | 2001:0db8:abcd:0000/64 |
| 4. Host bits | 128 − 64 = 64 bits |
| 5. Total host addresses | 2^64 = 18,446,744,073,709,551,616 |

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

- The matching routing table entry.

- The next-hop network/interface used to forward the packet.

- The default route that should be used if the destination does not match any routing table entry.

ANSWER :

The router has these routing entries:

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

Destination IP : 192.168.2.45

Destination Subnet

The destination IP is : 192.168.2.45

With /24, the first three octets represent the network : 192.168.2.0

Therefore : Destination subnet = 192.168.2.0/24

Matching Routing Table Entry

Compare the destination IP with the routing table:

192.168.1.0/24 → No match

192.168.2.0/24 → MATCH ✓

192.168.3.0/24 → No match

Therefore : Matching entry = 192.168.2.0/24

Next-Hop Network / Interface

The packet belongs to the 192.168.2.0/24 network.

Therefore, the router forwards the packet through the interface connected to the 192.168.2.0/24 customer network.

Next-hop network/interface = 192.168.2.0/24 interface

The question does not provide a specific next-hop IP address or interface name, so we should not invent one.

Default Route

If the destination does not match any of the listed routing entries, the router uses a default route.

The standard representation is : 0.0.0.0/0

It means : Match any IPv4 destination that has no more specific route in the routing table.

FINAL ANSWER :

| Requirement | Answer |
| --- | --- |
| 1. Destination Subnet | 192.168.2.0/24 |
| 2. Matching Routing Table Entry | 192.168.2.0/24 |
| 3. Next-Hop Network/Interface | Interface connected to 192.168.2.0/24 |
| 4. Default Route | 0.0.0.0/0 |

5. A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway that hosts in each LAN should use for communication.

ANSWER :

Given

Administration LAN  = 192.168.10.0/26

Finance LAN = 192.168.10.64/26

Administration gateway = 192.168.10.1

Finance Gateway = 192.168.10.65

Formulae Used

Host bits = 32 − Prefix length

Total addresses = 2^(Host bits)

Usable host addresses = Total addresses − 2

Block size = 256 − Last octet of subnet mask

For /26:

Subnet mask = 255.255.255.192

Host bits : 32 − 26 = 6 bits

Total addresses: 2^6 = 64 addresses

Usable host addresses: 64 − 2 = 62 hosts

Block size: 256 − 192 = 64

Therefore, each /26 subnet contains 64 addresses, with 62 usable host addresses.

Administration LAN – 192.168.10.0/26

The block size is 64, so the first subnet is: 192.168.10.0 – 192.168.10.63

Therefore:

- Network Address: 192.168.10.0

- Valid Host Range: 192.168.10.1 – 192.168.10.62

- Broadcast Address: 192.168.10.63

- Default Gateway: 192.168.10.1

Three Suitable Computers

Since .1 is already used as the gateway, suitable host addresses are:

- Computer 1 → 192.168.10.2

- Computer 2 → 192.168.10.3

- Computer 3 → 192.168.10.4

Finance LAN – 192.168.10.64/26

The next /26 block starts at .64: 192.168.10.64 – 192.168.10.127

Therefore:

- Network Address: 192.168.10.64

- Valid Host Range: 192.168.10.65 – 192.168.10.126

- Broadcast Address: 192.168.10.127

- Default Gateway: 192.168.10.65

Three Suitable Computers

Since .65 is already used as the gateway:

- Computer 1 → 192.168.10.66

- Computer 2 → 192.168.10.67

- Computer 3 → 192.168.10.68

FINAL ANSWER :

| LAN | Network Address | Valid Host IP Range | Broadcast Address | Default Gateway |
| --- | --- | --- | --- | --- |
| Administration | 192.168.10.0 | 192.168.10.1 – 192.168.10.62 | 192.168.10.63 | 192.168.10.1 |
| Finance | 192.168.10.64 | 192.168.10.65 – 192.168.10.126 | 192.168.10.127 | 192.168.10.65 |

| LAN | Computer 1 | Computer 2 | Computer 3 |
| --- | --- | --- | --- |
| Administration | 192.168.10.2 | 192.168.10.3 | 192.168.10.4 |
| Finance | 192.168.10.66 | 192.168.10.67 | 192.168.10.68 |

Usable hosts per LAN: 62

Administration Gateway: 192.168.10.1

Finance Gateway: 192.168.10.65

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context(2) | Shows good understanding with minor gaps(1.5) | Partial understanding; misses key aspects(1) | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved(2) | Identifies most key issues with minor omissions(1.5) | Identifies limited issues; some irrelevant points(1) | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario(2.5) | Applies concepts with minor errors(2) | Limited or partially correct application(1.5) | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale(2) | Makes reasonable decisions with some justification(1.5) | Makes weak decisions with limited justification(1) | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented(1.5) | Generally clear with minor issues(1) | Somewhat unclear or poorly structured(0.5) | Disorganized and difficult to understand |